You Lost your Helium Seed phrase

First of all. I very hate to tell you but if you lost the seed phrase or Solana private key, that’s it. There is no recovery. The app is very clear about this during setup. that is why it comes with a card to write your phrase down on. Take care of your seed phrases next time.

Read the YELLOW WARNING BOX here: https://docs.helium.com/wallets/wallet-seed-phrase

Requirements

- Lost Key SenseCAP M1

- Basic Linux Command Line

- Micro SD Card Reader

- Good Performance SD Card

- Time

Flash Raspberry Pi Image to Your SD Card with RPI Imager Download here https://www.raspberrypi.com/software/

Install and run it

Flash Your SD Card with Raspberry Pi Imager Choose Raspberry Pi 4 Board, Select OS Raspberry Pi OS (bookworm) 64bit (minimal or desktop is on your call) Setup User Name, Password, Wifi connection and Enable SSH

After Flashing and Verification Completed, Remove SD Card and insert to your SenseCAP M1 sd card slot

Use your favorite SSH application on your PC such as Terminus, Putty etc. then Connect to SenseCAP M1 IP address Enter your Username and Password

After Complete SSH Login

Configuration and Install

RPi Board Config

sudo raspi-config

Then Enable VNC, SPI, I2C and Serial port via Interface Menu of The configuration tool. Don't Reboot it Yet

Update RPi EEPROM

sudo rpi-eeprom-update -a

Increased Swap file to 2GB

sudo dphys-swapfile swapoff

sudo nano /etc/dphys-swapfile

Edit Following Line

CONF_SWAPSIZE=2048

And Hit Ctrl+X and Ctrl+Y then run

sudo dphys-swapfile setup

sudo dphys-swapfile swapon

Update to latest Kernel, Software and Lib

sudo apt update && sudo apt upgrade -y

Install Docker, Rootless Permission and Reboot

curl -sSL https://get.docker.com | sh && sudo usermod -aG docker $USER && sudo reboot

Clean Up APT Installation after Reboot

sudo apt autoremove -y

Clone Project Repo

git clone https://github.com/hereismeaw/SenseCAPM1-Chirpstack.git

Configure LoRaWAN EUID Of WM1302/1303 Module

Due to Debian 12 The GPIO sysfs is deprecated and superseded by libgpiod. In this repo this is my pre-compiled binary for RPi 4 for RPi OS Debian 12 based Works on WM1302 WM1303 module

cd SenseCAPM1-Chirpstack/packet_forwarder

sudo chmod +x reset_lgw.sh && sudo chmod +x lora_pkt_fwd && sudo chmod +x spectral_scan

./lora_pkt_fwd -c global_conf_as923_th.json

**For WM1303 Module use file "global_conf_as923_th_sx1303.json" instead

Watch For "Concentrator EUI" Line ex.

"INFO: concentrator EUI: 0xXXXXXXXXYYYYYYYY" Note It!

Edit LoRaWAN Driver Config

nano global_conf_as923_th.json

Look at Line 80

"gateway_ID": "AA555A0000000000",

Fill your "Concentrator EUI" And Hit Ctrl+X and Ctrl+Y

Create LoRaWAN Driver Systemd

sudo nano /etc/systemd/system/lora_pkt_fwd.service

Copy And Paste Following Line

[Unit]

Description=Southern IoT AS923 LoraWAN Packet Forwarder Version 01.07.2024 Copyrighted

After=network-online.target

Wants=network-online.target

[Service]

Type=simple

WorkingDirectory=/home/pi/SenseCAPM1-Chirpstack/packet_forwarder

ExecStart=/home/pi/SenseCAPM1-Chirpstack/packet_forwarder/lora_pkt_fwd -c /home/pi/SenseCAPM1-Chirpstack/packet_forwarder/global_conf_as923_th.json

Restart=always

RestartSec=30

StandardOutput=syslog

StandardError=syslog

SyslogIdentifier=lora_pkt_fwd

User=pi

[Install]

WantedBy=multi-user.target

Reload systemd Daemon

sudo systemctl daemon-reload

Start LoRaWAN Driver Now

sudo systemctl start lora_pkt_fwd.service

Check LoRaWAN Driver Status

sudo systemctl status lora_pkt_fwd.service

If no error so you are good to go!

Make LoRaWAN Driver Autostart On Boot

sudo systemctl enable lora_pkt_fwd.service

Install Chripstack Docker (This repo are using my pre configured AS923 Frequency for Thailand)

Before We Start. What is Chirpstack?

ChirpStack is an open-source LoRaWAN(R) Network Server which can be used to set up LoRaWAN networks. ChirpStack provides a web-interface for the management of gateways, devices and tenants as well to set up data integrations with the major cloud providers, databases and services commonly used for handling device data. ChirpStack provides a gRPC based API that can be used to integrate or extend ChirpStack.

cd ~/SenseCAPM1-Chirpstack/chirpstack-docker

docker compose up -d

Wait for 3-5 mins for Chirpstack to boot up then Browse to your SenseCAP M1 IP Address on port 8080

Default Login/Password : admin/admin ,, ex. 192.168.1.69:8080

Go to Gateways Menu on The Left, Add Your Gateway with Name, Gateway ID same as Packet Forwarder

(Optional) Import All LoRaWAN Device in Profile Template into Chirpstack

sudo make import-lorawan-devices

For Documentation and other binaries

Please refer to the ChirpStack website for documentation, API and pre-compiled binaries.

Install Node-Red and Start

cd ~/

bash <(curl -sL https://raw.githubusercontent.com/node-red/linux-installers/master/deb/update-nodejs-and-nodered)

Config Installation

Are you really sure you want to do this ? [y/N] ? y Would you like to install the Pi-specific nodes ? [y/N] ? y Running Node-RED install for user pi at /home/pi Settings file ‣ /home/pi/.node-red/settings.js (hit Enter)

Auto Start Node-Red On Boot

sudo systemctl enable nodered.service

Node Red running on port 1880, Use your favorite browser

ex. 192.168.1.69:1880

PS

- People Install Desktop Version. You can use your favorite VNC client to control desktop mode on your hotspot such as RealVNC etc.

For Advanced user who want to Connect to multiple LNS Simultaneously

Install ChirpStack Packet Multiplexer

The ChirpStack Packet Multiplexer utility forwards the Semtech packet-forwarder UDP data to multiple endpoints. It makes it possible to connect a single LoRa gateway to multiple networks. It is part of ChirpStack.

Ready? Go!

sudo apt install apt-transport-https dirmngr

sudo apt-key adv --keyserver keyserver.ubuntu.com --recv-keys 1CE2AFD36DBCCA00

sudo echo "deb https://artifacts.chirpstack.io/packages/3.x/deb stable main" | sudo tee /etc/apt/sources.list.d/chirpstack.list

sudo apt update && sudo apt install chirpstack-packet-multiplexer

To complete the installation, Make directory and update the configuration file which is located at

sudo mkdir /etc/chirpstack-packet-multiplexer

sudo nano /etc/chirpstack-packet-multiplexer/chirpstack-packet-multiplexer.toml

The Configuration Template I provided in this repo.

Bonus

You with this you can Install Mysterium node on your SenseCAP M1

What is a Mysterium Network Node? Network nodes are devices, such as this Raspberry Pi, which help power and maintain Mysterium distributed network. Any user of the network can pay to connect to your node, providing them with a VPN service, access to the open internet and a secure line of communication.

Run Following command

sudo -E bash -c "$(curl -s https://raw.githubusercontent.com/mysteriumnetwork/node/master/install.sh)"

In addition to downloading and installing our Node on your Raspberry Pi, this command will also install additional required dependencies like WireGuard if you don't have it already.

Reboot and Wait for 5-10 minutes for myst node to Start then browse to SenseCAP M1 IP Address with port 4449 in your favorite browser or use VNC connect to your hotspot ex.

192.168.1.69:4449